Lab Session: PID Tuning for Brushless DC Motor Speed Control

Course: Intelligent Physical System

Lab Duration: 2 hours

Prerequisite Knowledge: Students should be familiar with PID control basics, MATLAB, and BLDC motor dynamics.

Objective

This lab session aims to provide hands-on experience in tuning Proportional Integral Derivative (PID) controllers for the speed control of a Brushless DC Motor (BLDC). Students will explore both manual and automatic PID tuning methods, comparing their performance and understanding the impact of each gain parameter on the motor's response.

Overview

Students will use two MATLAB scripts:

- Manual PID Tuning: Students will iteratively adjust the 𝐾𝑝, 𝐾𝑖, 𝐾𝑑 values and observe the step response of the motor.

- Automatic PID Tuning: Students will explore automatic tuning techniques, including Ziegler-Nichols and MATLAB’s PID Tuner, to achieve optimal performance.

Equipment and Software

- MATLAB (with Control System Toolbox installed).

- Provided MATLAB scripts for manual and automatic PID tuning:

  - Manual_PID_DC_Tunning.m o 	Auto_PID_DC_Tunning.m

Lab Tasks

The lab is divided into two main tasks, focusing on manual and automatic PID tuning.

Task 1: Manual PID Tuning

Step 1: Initialize Parameters

- Load the Manual_PID_DC_Tunning.m script in MATLAB.

- Run the script to observe the initial step response with the default PID gains (𝐾𝑝 = 1, 𝐾𝑖 = 0, 𝐾𝑑 = 0

Step 2: Understand the Response

- Observe the initial response and identify characteristics such as rise time, overshoot, settling time, and steady-state error.

Step 3: Iteratively Adjust PID Gains

- Students will iteratively change 𝐾𝑝, 𝐾𝑖, 𝐾𝑑 values as prompted by the script.

- Adjust each gain parameter one at a time:

  - Start by increasing 𝐾𝑝 to observe its effect on the rise time and overshoot. o 	Next, increase 𝐾𝑖 to reduce the steady-state error but be cautious of integral windup.

  - Finally, adjust 𝐾𝑑 to reduce overshoot and improve system damping.

Step 4: Optimize the Response

- Continue tuning until the step response meets the following criteria:

  - Minimal overshoot. o 	Fast settling time.

  - Reduced steady-state error.

Step 5: Document Results

- Record the final PID gains that achieve the best performance.

- Plot and annotate the final step response, highlighting the improvements made through manual tuning.

Task 2: Automatic PID Tuning

Step 1: Run the Automatic PID Tuning Script

- Load the Auto_PID_DC_Tunning.m script in MATLAB.

- The script implements two automatic tuning methods:

  - Ziegler-Nichols Method

  - MATLAB’s PID Tuner

- Run the script and observe the response for each tuning method. Step 2: Analyze the Ziegler-Nichols Method

- Understand the Ziegler-Nichols method, which uses the estimated ultimate gain (𝐾𝑢) and ultimate period (𝑇𝑢)

- Observe how the automatically calculated gains (𝐾𝑝, 𝐾𝑖, 𝐾𝑑 ) compare with the manually tuned gains.

- Record the Ziegler-Nichols PID gains and plot the corresponding step response.

Step 3: Analyze MATLAB’s PID Tuner

- MATLAB’s PID Tuner provides an optimized set of gains using built-in tuning algorithms.

- Observe the step response and compare it with the Ziegler-Nichols and manually tuned responses.

Step 4: Compare Results

- Compare the performance of all three tuning methods (manual, Ziegler-Nichols, and MATLAB’s PID Tuner) based on:

  - Rise time. o 	Overshoot. o 	Settling time.

  - Steady-state error.

Step 5: Document Observations

- Create a table summarizing the PID gains and performance metrics for each method.

- Analyze which tuning method provided the best performance and discuss potential tradeoffs.

Lab Report Requirements

Students must submit a report that includes:

- Introduction: Brief explanation of the purpose of the lab and an overview of PID control.

- Methodology: Description of the manual and automatic tuning processes, including the parameters adjusted.

- Results:

  - Plots of step responses for each tuning method. o 	Comparison table of PID gains and performance metrics.

  - Figures and observations for task 1, task 2, and task 3. Your own version of the same figure in task 3. Shows both axises.

- Discussion:

  - Analysis of the effect of each PID gain on the BLDC motor’s speed response.

  - Explain your understanding of transfer functions, PID’s gains role, Ziegler-Nichols method.

O	Comparison of the tuning methods, discussing advantages and disadvantages of each.

- Conclusion: Summary of findings and lessons learned about PID tuning.

Learning Outcomes

By the end of this lab session, students will be able to:

- Understand the role of each PID gain in controlling a BLDC motor.

- Manually tune a PID controller to improve system performance.

- Apply automatic tuning methods and compare their effectiveness.

- Analyze and optimize control performance for a given setpoint.